RATEHD.PRG & RATEHD.DOC

Copyright 1989-1991  ICD, Inc.

This document and the program RATEHD.PRG, may be distributed by

any non-commercial means (may not be sold) as long as the they

are distributed together and no modifications are made.

Language translations are allowed.

The copyright holder is:

ICD, Inc.

1220 Rock Street

Rockford, IL  61101-1437

Voice:  (815) 968-2228

BBS:    (815) 968-2229

FAX:    (815) 968-6888

RATEHD.PRG was written in Personal Pascal, a product of ICD, Inc.

-----------------------------------------------------------------

RATEHD was written to show performance characteristics of

various hard disk drives.  It was developed at ICD to allow fair

comparison when evaluating hard disk drive mechanisms, embedded

SCSI drives and SCSI controllers when used on the Atari ST.  It

does not go through a hard disk handler (like ICDBOOT.SYS) so it

is not useful in rating the speed of different handlers.

Generally, the ICD hard disk handler provides a significant

speed improvement in actual use due to its caching.

We were a bit apprehensive about releasing this to the public

because of our fears that people would not understand it and

will use it for the wrong purpose.  It is intended as a

comparison test and should not be used to verify a drive or

controller against printed specifications.  Drive manufacturers

test their drives on specialized test equipment and not on an

Atari ST.  Data rates are also usually shown as a burst rate.

What we are measuring is the sustained rate, which is closer to

the top end in real life usage.

Our final decision to release RATEHD was twofold:

1) There is too much misinformation out there when it comes to

computers and hard drives.  There needed to be a uniform

benchmark test for the Atari ST.  Magazine reviews have

typically mislead the public, not maliciously, but because of

the reviewer's ignorance on the subject and lack of

scientific testing methods.  Now you have a benchmark; just

don't abuse it.

2) When we finished the FA-ST Tape Backup, we realized that a

hard drive system would need a certain amount of finesse to

keep up with the streaming tape.  If the tape had to keep

backing up and waiting for the hard drive to catch up, the

backup time would become laboriously long and almost not

usable.  A data rate of about 300 K/s (kilobytes per second)

is required to keep the tape moving without any stopping and

backing up.  As the data rate gets slower, the tape will begin

to backup and restart several times per partition.  A data

rate of less than 200 K/s would probably become unbearable at

attempting a backup.

We have tested many drives and controllers at their optimum

interleaves and found that Adaptec MFM controllers (including

Atari SH204, Megafile 20) run about 300 K/s if they are formatted

at 1:1 interleave.  Adaptec RLL controllers (including Atari

Megafile 30 and 60) run about 315 K/s at 2:1 interleave.  Omti

MFM controllers move along about 360 K/s using 1:1 with Omti RLL

at about 375 K/s at 1:1.  Embedded controller drives are usually

the speedsters with the Seagate ST138N and ST157N cruising at 550

K/s at 1:1.  The quickest we have measured was a Quantum LP105S

which flies at 1197 K/s which is probably fast enough to do real

time video.  It also had 21 ms average access time and had a 105

Megabyte formatted capacity.

THE PROBLEM WITH MOST TEST METHODS

----------------------------------

There are many variables in something as complex as a computer

with a hard disk drive that can make testing a cumbersome task.

The most obvious way to test hard drive speed is to copy files

from a hard drive to a RAMDISK and then back to a clean hard

drive while timing the process.  Although on the surface a test

like this appears sufficient, it cannot give an accurate picture

of what to expect in performance after using the drive for a

while.  Both hard drives would need to be freshly formatted (no

files yet) and with identical partition sizes.  If not freshly

formatted, the DOS fat table delays and fragmentation would enter

a significant amount of error into the rating equation.  Of

course the same DOS and same computer must also be used.

While this seems like a reasonable test method, it would only

show the data rate (assuming both drives were formatted at their

optimal interleave).  Step rate in this type of test would not be

a measurable factor as it would later on when the files on the

drives become fragmented.  Fragmentation occurs as a hard drive

gets used and files are modified.  They increase in size and

when rewritten to the drive, not all the file parts are in

consecutive sectors anymore, probably not even in the same area

of the disk.  The file eventually becomes a group of "fragments."

This is where the movement of the head from track to track (step

rate measured as average access time) really becomes an

important speed factor.  It would not be an easy task to create

two freshly formatted drives of different manufacture with

identical fragmentation on each drive.

Many of the magazine reviews we have seen give no indication of

the test method.  It is almost as if the numbers were arrived at

by magic.  We now offer RATEHD as a solution to all magazine

reviewers desiring to test hard disk drives.  If you all use

this, then the results can be believable.

Since RATEHD does not use the OS nor the hard disk handler, it

is easily repeatable and virtually foolproof.  Both are important

factors in any test method.  It will run on any hard drive and

does not care about how the partitions are set up nor does it

matter what the file structure is like.  Since RATEHD *never*

writes to the disk, it is totally safe and can be run at any

time.

ABOUT THE PROGRAM

-----------------

RATEHD begins at SCSI ID #0, Logical Unit Number 0, and proceeds

to check LUNs 0-3 for each SCSI ID going through all SCSI IDs 0-7

except special cases for ID #6 (reserved for our real time clock)

and ID# 7 (where it only reads LUN 0).  When the program finds a

hard drive present, it will print the manufacturers

identification under Device Name, and proceed with the tests.

After each drive is tested, it will display the results and

search for the next drive.  When finished with all drives, RATEHD

will allow you to RETEST or EXIT back to the desktop.  (A screen

dump will print the results.)

Don't worry: It is only a read test so no damage will be done to

your drive data.

ABOUT THE TESTS

---------------

The Data Rate Test

In this test, one megabyte of data is read (2,000 sectors)

in 100 sector continuous blocks (burst mode).  This is timed

and the math is done to calculate the number of K/s

(Kilobytes read in one second).

The Average Access Test

The Average Access is based on an assumed 16 megabyte

partition beginning with sector 0 and ending with sector

31,000.  The head is told to read a sector at the innermost

cylinder of the partition area (31,000) then to read one in

the outermost cylinder (0).  This is repeated 50 times but

each time the sector number is adjusted so that the head

will move 1/50 less distance. If there were 51 tries, the

head would end up on the center cylinder of the partition

(15,500), not stepping at all for the inner and outer

cylinders.

This movement is timed and the sector read time is removed.

An average time for the head travel is then calculated and

shown in ms (milliseconds).  Since this average time also

includes the command overhead time, it will be slightly

higher than specifications from the drive manufacturer.

WHAT YOU HAVE LEARNED

---------------------

There are really only two significant variables concerning

performance with a hard disk.  These are Data Rate and Average

Access.  We are assuming that you are concerned about speed and

have the optimum interleave on your drive.

There is one other bottleneck that does affect hard disk

performance.  The Disk Operating System is a major slowdown.  To

optimize your DOS speed get TOS 1.4 from Atari as soon as you

can.  Using it, we have seen a tremendous improvement in both

speed and reliability.  Thank you Atari!